# Arduino code

#include <Servo.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

const int trigPin = 9;

const int echoPin = 10;

const int servoPin = 6;

const int buzzerPin = 8;

const int ledPin = 7;

const int alertDistance = 20;

Servo radarServo;

long duration;

int distanceCm;

int getDistanceCm() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0) {

return 999; // no object / out of range

}

int distance = duration * 0.034 / 2;

if (distance > 100) {

distance = 100; // limit screen range

}

return distance;

}

void updateOLED(int angle, int distanceCm) {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("ULTRASONIC RADAR");

display.setCursor(0, 16);

display.print("Angle: ");

display.print(angle);

display.println(" deg");

display.setCursor(0, 28);

display.print("Distance: ");

if (distanceCm >= 100 || distanceCm == 999) {

display.println("Out");

} else {

display.print(distanceCm);

display.println(" cm");

}

display.setCursor(0, 44);

if (distanceCm <= alertDistance) {

display.setTextSize(2);

display.println("DANGER");

} else if (distanceCm <= 50) {

display.setTextSize(2);

display.println("WARN");

} else {

display.setTextSize(2);

display.println("CLEAR");

}

display.display();

}

void scanAtAngle(int angle) {

radarServo.write(angle);

delay(80);

distanceCm = getDistanceCm();

updateOLED(angle, distanceCm);

// Send data to Processing as: angle,distance.

Serial.print(angle);

Serial.print(",");

Serial.print(distanceCm);

Serial.println(".");

if (distanceCm <= 10) {

digitalWrite(ledPin, HIGH);

tone(buzzerPin, 1500);

} else if (distanceCm <= alertDistance) {

digitalWrite(ledPin, HIGH);

tone(buzzerPin, 1000);

} else if (distanceCm <= 50) {

digitalWrite(ledPin, HIGH);

tone(buzzerPin, 500);

delay(40);

noTone(buzzerPin);

} else {

digitalWrite(ledPin, LOW);

noTone(buzzerPin);

}

}

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(buzzerPin, OUTPUT);

pinMode(ledPin, OUTPUT);

radarServo.attach(servoPin);

Serial.begin(9600);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

while (true);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("Radar System");

display.setCursor(0, 16);

display.println("Starting...");

display.display();

delay(1000);

}

void loop() {

for (int angle = 15; angle <= 165; angle += 2) {

scanAtAngle(angle);

}

for (int angle = 165; angle >= 15; angle -= 2) {

scanAtAngle(angle);

}

}

# Java Processing code

import processing.serial.*;

Serial myPort;

String data = "";

int angle = 0;

int distance = 0;

void setup() {

size(900, 600);

smooth();

println(Serial.list());

// Change [0] to the correct port number if needed.

myPort = new Serial(this, "COM3", 9600);

myPort.bufferUntil('.');

}

void draw() {

background(0);

drawRadar();

drawLine();

drawObject();

drawText();

}

void serialEvent(Serial myPort) {

data = myPort.readStringUntil('.');

if (data != null) {

data = trim(data);

data = data.substring(0, data.length() - 1);

int commaIndex = data.indexOf(",");

if (commaIndex > 0) {

String angleString = data.substring(0, commaIndex);

String distanceString = data.substring(commaIndex + 1);

angle = int(angleString);

distance = int(distanceString);

}

}

}

void drawRadar() {

pushMatrix();

translate(width / 2, height - 80);

stroke(0, 255, 0);

noFill();

strokeWeight(2);

// Radar arcs

arc(0, 0, 800, 800, PI, TWO_PI);

arc(0, 0, 600, 600, PI, TWO_PI);

arc(0, 0, 400, 400, PI, TWO_PI);

arc(0, 0, 200, 200, PI, TWO_PI);

// Angle lines

for (int a = 0; a <= 180; a += 30) {

float x = 400 * cos(radians(a));

float y = -400 * sin(radians(a));

line(0, 0, x, y);

}

popMatrix();

}

void drawLine() {

pushMatrix();

translate(width / 2, height - 80);

stroke(0, 255, 0);

strokeWeight(3);

float x = 400 * cos(radians(angle));

float y = -400 * sin(radians(angle));

line(0, 0, x, y);

popMatrix();

}

void drawObject() {

if (distance >= 100) {

return;

}

pushMatrix();

translate(width / 2, height - 80);

stroke(255, 0, 0);

strokeWeight(8);

// Convert distance cm to screen pixels.

float pixDistance = map(distance, 0, 100, 0, 400);

float x = pixDistance * cos(radians(angle));

float y = -pixDistance * sin(radians(angle));

point(x, y);

popMatrix();

}

void drawText() {

fill(0, 255, 0);

textSize(20);

text("Mini Ultrasonic Radar System", 20, 30);

text("Angle: " + angle + "°", 20, 60);

if (distance >= 100) {

text("Distance: Out of range", 20, 90);

} else {

text("Distance: " + distance + " cm", 20, 90);

}

if (distance <= 20) {

fill(255, 0, 0);

text("ALERT: Object close!", 20, 120);

}

}